Project Abstract

Smart Insole for Early Detection of Diabetic Foot Ulcers Using IoT and AI

Problem Statement

Diabetic patients often suffer from foot ulcers caused by poor blood circulation, abnormal pressure distribution, and excessive skin moisture.

- These ulcers develop silently and, if not detected early, can lead to severe infections or even amputation.

- Traditional detection relies on manual checkups, which are not continuous and often too late.

- There is no affordable, portable, continuous monitoring solution for early ulcer prevention in low-resource settings.

Proposed Solution

We propose a smart wearable insole embedded with pressure, temperature, and skin-humidity sensors to monitor foot health in real-time.

- The insole continuously collects biometric data while the patient is walking or standing.

- A microcontroller with IoT connectivity (ESP32 / similar) processes and transmits data to a mobile app / cloud.

- AI algorithms analyze the patterns and detect early signs of:

  - Abnormal pressure points (risk of sores).

  - Rising skin temperature (possible infection).

  - High skin moisture (favorable condition for bacterial growth).

When abnormal conditions are detected, the system alerts the user and caregiver with early warnings, preventing complications.

System Components

- Sensors

  - Force Sensing Resistors (FSR) → 5 per insole, to measure pressure distribution (10 kg each).

  - Digital Temperature Sensor (DS18B20 or MLX90614 IR) → for localized skin temperature monitoring.

  - Capacitive Skin Humidity Sensor (DFRobot Capacitive Moisture Module / FDC1004 with electrodes) → to measure local moisture.

  - (Optional) Ambient humidity (BME280) for shoe environment analysis.

- Processing & Communication

  - ESP32 microcontroller (low-cost, WiFi+Bluetooth).

  - Power supply: Rechargeable Li-ion coin battery with charging circuit.

- Software & AI

  - Local preprocessing on ESP32.

  - AI/ML model (trained on pressure/temperature/humidity patterns) for anomaly detection.

  - Mobile app for real-time monitoring, alerts, and logs.

Working Principle

- Sensors embedded in the insole continuously record pressure, temperature, and moisture data.

- Data is transmitted via Bluetooth/WiFi to a mobile app.

- AI model compares readings with user’s baseline profile and medical thresholds.

- When early warning signs appear (hotspot, excessive wetness, abnormal load), the system triggers an alert.

- Data logs are shared with caregivers/doctors for preventive treatment.

Innovative Features

- Portable & wearable → fits inside regular shoes.

- Low-cost → uses commercially available sensors.

- AI-driven personalized alerts (adaptive per user).

- Cloud + app integration for doctors & caregivers.

- Prevents severe complications → reduces risk of amputation.

Commercial Potential

- Target users: Diabetic patients, elderly, hospitals, rural clinics.

- Market gap: Current solutions are expensive (>₹20,000), bulky, and not accessible in India.

- Affordable version: Prototype cost ~₹3,000–₹4,000; final optimized product could retail at ₹6,000–₹8,000.

- Business model: Direct sales + subscription for app analytics.

✅ Impact: This smart insole can save lives and reduce healthcare costs by enabling early detection of diabetic foot ulcers. It addresses a specific, critical medical need using IoT, AI, embedded systems, and sensors — making it both innovative and commercially viable.