give fulll circuit connections with pins

Here is your complete circuit connection documentation for the Smart Diabetic Foot Ulcer Monitoring System (Both Shoes – ESP32 + FSR + Temperature Sensor). You can directly include this in your Hardware / Implementation chapter.

FULL CIRCUIT CONNECTIONS WITH PIN DETAILS

1. Components Used

- ESP32 (2 units – Left & Right Shoe)

- FSR Sensors (4 per shoe → total 8)

- Resistors (4.7kΩ × 8)

- DS18B20 Temperature Sensor (2)

- 4.7kΩ pull-up resistor (for DS18B20)

- Jumper wires

- Power supply (3.3V from ESP32)

2. FSR SENSOR CONNECTION (FOR ONE SHOE)

FSR Working Circuit (Voltage Divider)

4

Connection Explanation

Each FSR must be connected using a voltage divider circuit.

General Connection Formula

3.3V ---- FSR ---- Analog Pin ---- 4.7kΩ ---- GND

LEFT SHOE – FSR PIN CONNECTIONS

| Sensor Position | ESP32 Pin | Connection |
| --- | --- | --- |
| Heel | GPIO 32 | Analog Input |
| Midfoot | GPIO 33 | Analog Input |
| Ball | GPIO 34 | Analog Input |
| Toe | GPIO 35 | Analog Input |

RIGHT SHOE – FSR PIN CONNECTIONS

| Sensor Position | ESP32 Pin | Connection |
| --- | --- | --- |
| Heel | GPIO 35 | Analog Input |
| Midfoot | GPIO 33 | Analog Input |
| Ball | GPIO 34 | Analog Input |
| Toe | GPIO 32 | Analog Input |

3. DS18B20 TEMPERATURE SENSOR CONNECTION

Pin Configuration of DS18B20

| DS18B20 Pin | Connection |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| DATA | GPIO 25 |

Important

- Add 4.7kΩ resistor between DATA and VCC

- This is required for OneWire communication

4. POWER CONNECTIONS

| Component | Connection |
| --- | --- |
| ESP32 | USB / Battery |
| FSR Sensors | 3.3V |
| DS18B20 | 3.3V |

5. COMPLETE CONNECTION SUMMARY (ONE SHOE)

ESP32 Pins Used

| Function | Pin |
| --- | --- |
| FSR Heel | GPIO 32 |
| FSR Midfoot | GPIO 33 |
| FSR Ball | GPIO 34 |
| FSR Toe | GPIO 35 |
| Temperature Sensor | GPIO 25 |
| Power | 3.3V |
| Ground | GND |

6. BOTH SHOES ARCHITECTURE

4

System Flow

LEFT SHOE ESP32 → WiFi → Python Server
RIGHT SHOE ESP32 → WiFi → Python Server

7. IMPORTANT PRACTICAL NOTES

✔ Use ADC Pins Only

- GPIO 32, 33, 34, 35 are analog pins

- Do NOT use digital-only pins

✔ Common Ground Required

- All sensors must share same GND

✔ Stable Readings Tip

- Use 4.7kΩ resistor (not 10kΩ)

- This improves sensitivity

✔ Avoid Floating Values

- Always use proper resistor

- Loose wiring causes random values

8. FINAL CONNECTION DIAGRAM (TEXT FORMAT)

FSR (each sensor):
3.3V → FSR → ESP32 Analog Pin → 4.7kΩ → GND

DS18B20:
VCC → 3.3V
GND → GND
DATA → GPIO 25
(4.7kΩ between DATA & VCC)

Conclusion

This circuit setup enables:

- Accurate foot pressure sensing

- Reliable skin temperature measurement

- Stable data transmission via ESP32 WiFi